🌐 CCNA Lab 03 – Directly Attached Static Routing

📌 Lab Title

Implementation of Directly Attached Static Routing Between Two Networks

🎯 Lab Objective

The purpose of this lab is to learn how to configure Directly Attached Static Routes on Cisco routers to enable communication between two remote LAN networks.

After completing this lab, you will be able to:

- Configure basic router settings.

- Assign IP addresses to router interfaces.

- Connect multiple LANs through routers.

- Configure directly attached static routes using exit interfaces.

- Verify routing table entries.

- Test end-to-end network connectivity.

🛠️ Devices Used

| Device | Quantity |
| --- | --- |
| Cisco 1841 Router | 2 |
| Cisco 2960 Switch | 2 |
| PC | 4 |
| Copper Straight Through Cable | 6 |
| Serial DCE Cable | 1 |

📊 Network Topology

LAN 1 (192.168.1.0/24)

PC1                PC2

|                  |

|                  |

+------ SW1 -------+

|

|

Fa0/0|

R1

192.168.1.1

S0/0/0 192.168.3.1

||

||

S0/0/0 192.168.3.2

R2

192.168.2.1

Fa0/0|

|

|

+------ SW2 -------+

|                  |

|                  |

PC3                PC4

LAN 2 (192.168.2.0/24)

📋 IP Addressing Table

Router R1

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| Fa0/0 | 192.168.1.1 | 255.255.255.0 |
| S0/0/0 | 192.168.3.1 | 255.255.255.252 |

Router R2

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| Fa0/0 | 192.168.2.1 | 255.255.255.0 |
| S0/0/0 | 192.168.3.2 | 255.255.255.252 |

PCs

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC1 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 | 192.168.1.20 | 255.255.255.0 | 192.168.1.1 |
| PC3 | 192.168.2.10 | 255.255.255.0 | 192.168.2.1 |
| PC4 | 192.168.2.20 | 255.255.255.0 | 192.168.2.1 |

🔌 Physical Connections

Switch 1 (LAN 1)

| Device | Connected To |
| --- | --- |
| PC1 | SW1 Fa0/1 |
| PC2 | SW1 Fa0/2 |
| R1 Fa0/0 | SW1 Fa0/24 |

Switch 2 (LAN 2)

| Device | Connected To |
| --- | --- |
| PC3 | SW2 Fa0/1 |
| PC4 | SW2 Fa0/2 |
| R2 Fa0/0 | SW2 Fa0/24 |

Router-to-Router Connection

| Device | Interface |
| --- | --- |
| R1 | S0/0/0 |
| R2 | S0/0/0 |

Network:

192.168.3.0/30

🔹 Step 1: Configure Basic Settings on R1

enable

configure terminal

hostname R1

enable secret cisco123

line console 0

password console123

login

exit

line vty 0 4

password vty123

login

exit

service password-encryption

🔹 Step 2: Configure Interfaces on R1

interface fastethernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface serial 0/0/0

ip address 192.168.3.1 255.255.255.252

clock rate 64000

no shutdown

exit

🔹 Step 3: Configure Basic Settings on R2

enable

configure terminal

hostname R2

enable secret cisco123

line console 0

password console123

login

exit

line vty 0 4

password vty123

login

exit

service password-encryption

🔹 Step 4: Configure Interfaces on R2

interface fastethernet 0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface serial 0/0/0

ip address 192.168.3.2 255.255.255.252

no shutdown

exit

🔹 Step 5: Configure Directly Attached Static Routes

Configure Route on R1

To reach LAN 2 (192.168.2.0/24), send traffic out Serial0/0/0.

R1(config)# ip route 192.168.2.0 255.255.255.0 serial 0/0/0

Configure Route on R2

To reach LAN 1 (192.168.1.0/24), send traffic out Serial0/0/0.

R2(config)# ip route 192.168.1.0 255.255.255.0 serial 0/0/0

🔹 Step 6: Verify Interface Status

show ip interface brief

Expected Output:

Interface      IP-Address      Status      Protocol

Fa0/0          192.168.1.1     up          up

S0/0/0         192.168.3.1     up          up

🔹 Step 7: Verify Routing Table

On Router R1

show ip route

Expected Output:

S 192.168.2.0/24 is directly connected, Serial0/0/0

On Router R2

show ip route

Expected Output:

S 192.168.1.0/24 is directly connected, Serial0/0/0

🔹 Step 8: Test Connectivity

Test Router-to-Router Communication

From R1:

ping 192.168.3.2

Expected Result:

!!!!!

Test End-to-End Communication

From PC1:

ping 192.168.2.10

Expected Result:

Reply from 192.168.2.10

From PC2:

ping 192.168.2.20

From PC3:

ping 192.168.1.10

From PC4:

ping 192.168.1.20

All pings should be successful.

🔹 Step 9: Save Configuration

On R1

copy running-config startup-config

On R2

copy running-config startup-config

📚 Commands Learned

hostname

enable secret

line console 0

line vty 0 4

password

login

service password-encryption

interface

ip address

no shutdown

clock rate

ip route

show ip route

show ip interface brief

ping

copy running-config startup-config

🎓 Concepts Learned

✅ Basic Router Configuration

✅ Interface Configuration

✅ Serial WAN Configuration

✅ IP Addressing

✅ Static Routing

✅ Directly Attached Static Routes

✅ Routing Table Verification

✅ End-to-End Connectivity Testing

✅ Network Troubleshooting

📝 Lab Summary

In this lab, two Cisco routers, two switches, and four PCs were configured to create two separate LAN networks. Router R1 served the 192.168.1.0/24 network, while Router R2 served the 192.168.2.0/24 network. Both routers were connected through a serial WAN link using the 192.168.3.0/30 network. Directly Attached Static Routes were configured by specifying the outgoing serial interface instead of a next-hop IP address. After verifying the routing tables and testing connectivity with ping commands, successful communication was established between all hosts on both LANs. This lab demonstrated how directly attached static routing enables communication between remote networks using exit interfaces.