🌐 CCNA Lab 08 – Multi-Area OSPF Configuration

📌 Lab Title

Configuration of Multi-Area OSPF Using Area 0, Area 50, and Area 100

🎯 Lab Objective

The objective of this lab was to configure Multi-Area OSPF using:

- OSPF Process ID 1

- Area 0 (Backbone Area)

- Area 50

- Area 100

The main goal achieved in this lab was:

✅ Configure Multi-Area OSPF

✅ Configure Router IDs manually

✅ Establish OSPF Neighbor Relationships

✅ Exchange Routing Information Between Areas

✅ Verify Inter-Area Routes

✅ Successfully Ping from LAN 1 PC to LAN 2 PC

📖 Introduction to Multi-Area OSPF

What is Multi-Area OSPF?

Multi-Area OSPF divides a large network into multiple areas.

Instead of placing every router in a single area, the network is separated into logical areas to improve:

- Scalability

- Performance

- Faster Convergence

- Reduced Routing Updates

OSPF Areas Used

Area 0

Backbone Area

All OSPF areas must connect to Area 0.

Area 50

Left Side LAN Area

Contains:

- R1

- R3

- LAN 1

Area 100

Right Side LAN Area

Contains:

- R2

- R4

- LAN 2

🛠️ Devices Used

| Device | Quantity |
| --- | --- |
| Cisco 2901 Router | 4 |
| Cisco 2960 Switch | 2 |
| PC | 2 |
| Copper Cables | 6 |

📊 Network Topology

Area 0 (Backbone)

R1 ---------------- R2

|                  |

|                  |

|                  |

Area 50           Area 100

|                  |

|                  |

R3                R4

|                  |

|                  |

LAN1              LAN2

🆔 Router IDs

| Router | Router ID |
| --- | --- |
| R1 | 1.1.1.1 |
| R2 | 2.2.2.2 |
| R3 | 3.3.3.3 |
| R4 | 4.4.4.4 |

📋 IP Addressing Scheme

LAN 1

| Device | IP Address |
| --- | --- |
| PC1 | 192.168.1.10/24 |
| R3 G0/1 | 192.168.1.1/24 |

Area 50 Link

| Device | IP Address |
| --- | --- |
| R1 G0/1 | 192.168.50.1/24 |
| R3 G0/0 | 192.168.50.2/24 |

Area 0 Link

| Device | IP Address |
| --- | --- |
| R1 G0/0 | 192.168.0.1/24 |
| R2 G0/0 | 192.168.0.2/24 |

Area 100 Link

| Device | IP Address |
| --- | --- |
| R2 G0/1 | 192.168.100.1/24 |
| R4 G0/0 | 192.168.100.2/24 |

LAN 2

| Device | IP Address |
| --- | --- |
| PC2 | 192.168.2.10/24 |
| R4 G0/1 | 192.168.2.1/24 |

🔹 Step 1: Basic Router Configuration

Perform on all routers.

R1

enable

configure terminal

hostname R1

enable secret cisco123

line console 0

password console123

login

service password-encryption

Repeat for R2, R3 and R4.

🔹 Step 2: Configure Interfaces

R1

interface g0/0

ip address 192.168.0.1 255.255.255.0

no shutdown

interface g0/1

ip address 192.168.50.1 255.255.255.0

no shutdown

R2

interface g0/0

ip address 192.168.0.2 255.255.255.0

no shutdown

interface g0/1

ip address 192.168.100.1 255.255.255.0

no shutdown

R3

interface g0/0

ip address 192.168.50.2 255.255.255.0

no shutdown

interface g0/1

ip address 192.168.1.1 255.255.255.0

no shutdown

R4

interface g0/0

ip address 192.168.100.2 255.255.255.0

no shutdown

interface g0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

🔹 Step 3: Configure OSPF Process ID 1

R1

router ospf 1

router-id 1.1.1.1

network 192.168.0.0 0.0.0.255 area 0

network 192.168.50.0 0.0.0.255 area 50

Explanation

- Connected to Backbone Area 0

- Connected to Area 50

- Functions as ABR (Area Border Router)

R2

router ospf 1

router-id 2.2.2.2

network 192.168.0.0 0.0.0.255 area 0

network 192.168.100.0 0.0.0.255 area 100

Explanation

- Connected to Backbone Area 0

- Connected to Area 100

- Functions as ABR

R3

router ospf 1

router-id 3.3.3.3

network 192.168.50.0 0.0.0.255 area 50

network 192.168.1.0 0.0.0.255 area 50

Explanation

Internal Router of Area 50.

R4

router ospf 1

router-id 4.4.4.4

network 192.168.100.0 0.0.0.255 area 100

network 192.168.2.0 0.0.0.255 area 100

Explanation

Internal Router of Area 100.

🔹 Step 4: Verify OSPF Neighbors

On R1

show ip ospf neighbor

Expected:

Neighbor ID     State

2.2.2.2         FULL

3.3.3.3         FULL

On R2

show ip ospf neighbor

Expected:

Neighbor ID     State

1.1.1.1         FULL

4.4.4.4         FULL

🔹 Step 5: Verify OSPF Routes

show ip route ospf

Expected Routes on R3:

O IA 192.168.2.0/24

O IA 192.168.100.0/24

Meaning

O IA = OSPF Inter-Area Route

The routes are learned from another OSPF area through Area 0.

🔹 Step 6: Verify OSPF Database

show ip ospf database

Purpose:

- Verify LSAs.

- Verify Area Information.

- Verify Router Advertisements.

🔹 Step 7: Test Connectivity

From LAN 1 PC

ping 192.168.2.10

Result:

!!!!!

Successful.

Reverse Test

From LAN 2 PC

ping 192.168.1.10

Result:

!!!!!

Successful.

🔹 Step 8: Save Configuration

copy running-config startup-config

Run on all routers.

📚 Verification Commands Used

show ip ospf neighbor

show ip route

show ip route ospf

show ip ospf database

show running-config

ping

🎓 Concepts Learned

✅ OSPF Multi-Area Design

✅ Area 0 Backbone Configuration

✅ Area 50 Configuration

✅ Area 100 Configuration

✅ Router ID Configuration

✅ Area Border Router (ABR)

✅ OSPF Neighbor Relationships

✅ OSPF Inter-Area Routing

✅ Route Advertisement Between Areas

✅ End-to-End Connectivity Testing

📝 Lab Summary

In this lab, Multi-Area OSPF was configured using Process ID 1. Four routers were assigned Router IDs (1.1.1.1 to 4.4.4.4) and organized into three OSPF areas: Area 0 (Backbone Area), Area 50, and Area 100. R1 and R2 acted as Area Border Routers (ABRs) connecting the non-backbone areas to Area 0. OSPF neighbor relationships were successfully established, routing information was exchanged between areas, and inter-area routes were learned dynamically. The primary objective of the lab was achieved when PC1 in LAN 1 successfully pinged PC2 in LAN 2, proving successful communication across multiple OSPF areas.