🌐 CCNA Lab 13 – Extended ACL with IPv4, DHCP, and EIGRP

📌 Lab Title

Configuring Extended Access Control List (ACL), DHCP, IPv4 Addressing, and EIGRP AS 50

🎯 Lab Objective

The objective of this lab was to configure an Extended Access Control List (ACL) to control communication between specific hosts in two different LANs.

The network was configured with:

- IPv4 Addressing

- DHCP for LAN 1 and LAN 2

- EIGRP AS 50 Dynamic Routing

- Extended ACL for traffic filtering

The primary goal achieved in this lab was:

✅ Configure IPv4 Addressing

✅ Configure DHCP Pools

✅ Configure EIGRP AS 50

✅ Verify End-to-End Connectivity

✅ Configure an Extended ACL

✅ Prevent PC1 (192.168.1.2) from pinging PC4 (192.168.2.3)

✅ Allow all other traffic

📖 Introduction to Extended ACL

What is an Extended ACL?

An Extended Access Control List (Extended ACL) is used to filter network traffic based on multiple criteria, making it much more powerful than a Standard ACL.

Unlike a Standard ACL, which only checks the source IP address, an Extended ACL can examine:

- Source IP Address

- Destination IP Address

- Protocol (ICMP, TCP, UDP, etc.)

- Port Numbers (HTTP, HTTPS, FTP, SSH, etc.)

This allows administrators to create very specific security policies.

Difference Between Standard ACL and Extended ACL

| Standard ACL | Extended ACL |
| --- | --- |
| Filters only the source IP address | Filters source IP, destination IP, protocol, and ports |
| Less flexible | Highly flexible |
| Usually placed close to the destination | Usually placed close to the source |
| Number range: 1–99 and 1300–1999 | Number range: 100–199 and 2000–2699 |

Why Use an Extended ACL?

Suppose there are two users:

- PC1 (192.168.1.2)

- PC2 (192.168.1.3)

You want:

- PC1 cannot ping PC4 (192.168.2.3)

- PC2 can ping PC4

- All other communication remains allowed

A Standard ACL cannot achieve this because it filters only on the source address. An Extended ACL can match both the source and the destination, making it the correct solution.

ACL Processing Rules

ACL entries are processed from top to bottom.

The router checks each packet against the ACL statements in order.

- The first matching statement is applied.

- If no statement matches, an implicit deny is applied automatically.

Example:

permit tcp any any eq 80

deny icmp host 192.168.1.2 host 192.168.2.3

permit ip any any

Best Practice

Extended ACLs should be placed as close as possible to the source of the traffic. This prevents unwanted traffic from traveling across the network.

🛠️ Devices Used

| Device | Quantity |
| --- | --- |
| Cisco 1841 Routers | 2 |
| Cisco 2960 Switches | 2 |
| PCs | 4 |
| Copper Straight-Through Cables | 6 |

📊 Network Topology

EIGRP AS 50

192.168.3.0/24

R1 ---------------- R2

|                  |

|                  |

192.168.1.0/24      192.168.2.0/24

|                  |

SW1                SW2

/   \              /   \

PC1   PC2          PC3   PC4

📋 IP Addressing Scheme

| Network | Address |
| --- | --- |
| LAN 1 | 192.168.1.0/24 |
| LAN 2 | 192.168.2.0/24 |
| R1–R2 Link | 192.168.3.0/24 |

Router Interfaces

R1

| Interface | IP Address |
| --- | --- |
| G0/0 | 192.168.3.1/24 |
| G0/1 | 192.168.1.1/24 |

R2

| Interface | IP Address |
| --- | --- |
| G0/0 | 192.168.3.2/24 |
| G0/1 | 192.168.2.1/24 |

🔹 Step 1: Basic Router Configuration

Configure the hostname, enable secret password, console password, and interface IP addresses on both routers.

🔹 Step 2: Configure IPv4 Addresses

Configure all router interfaces with the IPv4 addresses shown in the IP addressing table and enable them using the no shutdown command.

🔹 Step 3: Configure DHCP Pools

R1 – LAN 1

ip dhcp excluded-address 192.168.1.1

ip dhcp pool LAN1

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1

dns-server 8.8.8.8

domain-name ccnalab.com

R2 – LAN 2

ip dhcp excluded-address 192.168.2.1

ip dhcp pool LAN2

network 192.168.2.0 255.255.255.0

default-router 192.168.2.1

dns-server 8.8.8.8

domain-name ccnalab.com

🔹 Step 4: Configure EIGRP AS 50

R1

router eigrp 50

no auto-summary

network 192.168.1.0 0.0.0.255

network 192.168.3.0 0.0.0.255

R2

router eigrp 50

no auto-summary

network 192.168.2.0 0.0.0.255

network 192.168.3.0 0.0.0.255

🔹 Step 5: Verify Connectivity

Before applying the ACL, verify that all hosts can communicate.

From PC1:

ping 192.168.2.3

Expected result:

!!!!!

Successful.

This confirms that routing is working correctly before traffic filtering is applied.

🔹 Step 6: Configure the Extended ACL

The objective is to deny only ICMP (ping) from PC1 (192.168.1.2) to PC4 (192.168.2.3) while allowing all other traffic.

On R1, create the ACL:

access-list 100 deny icmp host 192.168.1.2 host 192.168.2.3

access-list 100 permit ip any any

Explanation

- deny icmp blocks only ping traffic.

- host 192.168.1.2 specifies the source PC.

- host 192.168.2.3 specifies the destination PC.

- permit ip any any allows all remaining IP traffic.

🔹 Step 7: Apply the ACL

Because this is an Extended ACL, it should be applied close to the source.

Apply it to the LAN interface of R1:

interface g0/1

ip access-group 100 in

Why Inbound?

Traffic enters R1 through G0/1 from LAN 1. Filtering it here prevents unwanted packets from crossing the network.

🔹 Step 8: Verify the ACL

Display the ACL:

show access-lists

Expected output:

Extended IP access list 100

deny icmp host 192.168.1.2 host 192.168.2.3

permit ip any any

Verify the interface:

show ip interface g0/1

Expected output:

Inbound access list is 100

🔹 Step 9: Test Connectivity After ACL

PC1 → PC4

ping 192.168.2.3

Expected Result:

Request timed out

❌ Ping blocked.

PC1 → Router R2

ping 192.168.3.2

Expected Result:

!!!!!

✅ Successful.

PC2 → PC4

ping 192.168.2.3

Expected Result:

!!!!!

✅ Successful.

PC3 → PC1

ping 192.168.1.2

Expected Result:

!!!!!

✅ Successful (the ACL blocks only ICMP from PC1 to PC4).

🔹 Step 10: Save Configuration

copy running-config startup-config

Run this command on both routers.

📚 Verification Commands

show ip route

show ip eigrp neighbors

show ip protocols

show access-lists

show ip interface g0/1

show ip dhcp binding

show ip dhcp pool

ping

🎓 Concepts Learned

✅ IPv4 Address Configuration

✅ DHCP Server Configuration

✅ EIGRP AS 50 Configuration

✅ Extended ACL Configuration

✅ ICMP Traffic Filtering

✅ Source and Destination-Based Filtering

✅ Proper ACL Placement (Close to the Source)

✅ ACL Verification Commands

✅ End-to-End Connectivity Testing

📝 Lab Summary

In this lab, two Cisco routers were configured with IPv4 addressing, DHCP services, and EIGRP AS 50 to provide dynamic routing between two LANs. After confirming successful connectivity, an Extended ACL (100) was created on R1 to deny ICMP (ping) traffic from PC1 (192.168.1.2) to PC4 (192.168.2.3) while allowing all other IP traffic. The ACL was applied inbound on R1's G0/1 interface, following the best practice of placing Extended ACLs close to the source. After implementation, PC1 could no longer ping PC4, while other hosts and services continued to communicate normally. This lab demonstrated how Extended ACLs provide precise traffic filtering based on the source address, destination address, and protocol type.