🌐 CCNA Lab 05 – RIP Routing (RIPv2)

📌 Lab Title

Configuration of RIP Version 2 Routing Protocol

🎯 Lab Objective

The objective of this lab is to configure Routing Information Protocol Version 2 (RIPv2) on multiple routers to enable dynamic routing between remote networks.

In this lab, all networks use the /24 prefix (255.255.255.0), and routers automatically learn remote routes through RIP advertisements.

🛠️ Devices Used

| Device | Quantity |
| --- | --- |
| Cisco 1841 Router | 4 |
| Cisco 2960 Switch | 2 |
| PC | 4 |
| Serial DCE/DTE Cables | 4 |
| Copper Straight Through Cables | 6 |

📊 Network Topology

R2

192.168.2.0/24

/        \

/          \

/            \

/              \

R1                R4

\              /

\            /

\          /

\        /

192.168.3.0/24

R3

LAN 1                                 LAN 2

192.168.1.0/24                    192.168.6.0/24

📋 IP Addressing Scheme

Router R1

| Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| Fa0/0 | 192.168.1.1 | 255.255.255.0 |
| S0/0/0 | 192.168.2.1 | 255.255.255.0 |
| S0/0/1 | 192.168.3.1 | 255.255.255.0 |

Router R2

| Interface | IP Address |
| --- | --- |
| S0/0/0 | 192.168.2.2 |
| S0/0/1 | 192.168.4.1 |

Subnet Mask:

255.255.255.0

Router R3

| Interface | IP Address |
| --- | --- |
| S0/0/0 | 192.168.3.2 |
| S0/0/1 | 192.168.5.1 |

Subnet Mask:

255.255.255.0

Router R4

| Interface | IP Address |
| --- | --- |
| Fa0/0 | 192.168.6.1 |
| S0/0/0 | 192.168.5.2 |
| S0/0/1 | 192.168.4.2 |

Subnet Mask:

255.255.255.0

🖥️ PC Configuration

LAN 1

PC0

IP Address : 192.168.1.10

Subnet Mask: 255.255.255.0

Gateway    : 192.168.1.1

PC1

IP Address : 192.168.1.20

Subnet Mask: 255.255.255.0

Gateway    : 192.168.1.1

LAN 2

PC2

IP Address : 192.168.6.10

Subnet Mask: 255.255.255.0

Gateway    : 192.168.6.1

PC3

IP Address : 192.168.6.20

Subnet Mask: 255.255.255.0

Gateway    : 192.168.6.1

🔹 Step 1: Basic Configuration on All Routers

Perform the following on R1, R2, R3, and R4:

enable

configure terminal

hostname R1

enable secret cisco123

line console 0

password console123

login

exit

service password-encryption

Repeat and change hostname accordingly:

R1

R2

R3

R4

🔹 Step 2: Configure Interfaces

R1

interface fa0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

interface s0/0/0

ip address 192.168.2.1 255.255.255.0

clock rate 64000

no shutdown

interface s0/0/1

ip address 192.168.3.1 255.255.255.0

clock rate 64000

no shutdown

R2

interface s0/0/0

ip address 192.168.2.2 255.255.255.0

no shutdown

interface s0/0/1

ip address 192.168.4.1 255.255.255.0

no shutdown

R3

interface s0/0/0

ip address 192.168.3.2 255.255.255.0

no shutdown

interface s0/0/1

ip address 192.168.5.1 255.255.255.0

no shutdown

R4

interface fa0/0

ip address 192.168.6.1 255.255.255.0

no shutdown

interface s0/0/0

ip address 192.168.5.2 255.255.255.0

no shutdown

interface s0/0/1

ip address 192.168.4.2 255.255.255.0

no shutdown

🔹 Step 3: Configure RIP Version 2

R1

router rip

version 2

no auto-summary

network 192.168.1.0

network 192.168.2.0

network 192.168.3.0

R2

router rip

version 2

no auto-summary

network 192.168.2.0

network 192.168.4.0

R3

router rip

version 2

no auto-summary

network 192.168.3.0

network 192.168.5.0

R4

router rip

version 2

no auto-summary

network 192.168.4.0

network 192.168.5.0

network 192.168.6.0

🔹 Step 4: Verify RIP Neighbors

show ip protocols

Expected Output:

Routing Protocol is "rip"

Sending updates every 30 seconds

Version control: send version 2

🔹 Step 5: Verify Routing Table

On R1:

show ip route

Expected RIP routes:

R 192.168.4.0

R 192.168.5.0

R 192.168.6.0

On R4:

show ip route

Expected:

R 192.168.1.0

R 192.168.2.0

R 192.168.3.0

🔹 Step 6: Test Connectivity

From PC0:

ping 192.168.6.10

Expected:

!!!!!

From PC1:

ping 192.168.6.20

From PC2:

ping 192.168.1.10

From PC3:

ping 192.168.1.20

All pings should be successful.

🔹 Step 7: Verify RIP Learned Routes

show ip route rip

Expected Output:

R    192.168.4.0/24

R    192.168.5.0/24

R    192.168.6.0/24

The letter R indicates routes learned through RIP.

🔹 Step 8: Save Configuration

On every router:

copy running-config startup-config

📚 Commands Learned

router rip

version 2

no auto-summary

network

show ip protocols

show ip route

show ip route rip

copy running-config startup-config

🎓 Concepts Learned

✅ Dynamic Routing

✅ RIP Version 2

✅ Route Advertisement

✅ Route Learning

✅ Routing Table Verification

✅ Multi-Path Network Design

✅ RIP Timers

✅ No Auto-Summary

✅ End-to-End Connectivity Testing

📝 Lab Summary

In this lab, four Cisco routers were configured in a redundant topology and connected using six different /24 networks. RIP Version 2 was enabled on all routers, allowing them to dynamically exchange routing information. The no auto-summary command was used to support classless routing. After RIP converged, all routers automatically learned remote networks without manually configuring static routes. Connectivity between the 192.168.1.0/24 LAN and the 192.168.6.0/24 LAN was successfully verified using ping tests. This lab demonstrated how RIPv2 simplifies route management by dynamically sharing network information among routers.